CISCO Network Fundamentals Online Course V. 4.0 

Summary Chapter 4
OSI Transport Layer.

4.1.1 Purpose of the Transport Layer
The Transport layer provides for the segmentation of data and the control necessary to reassemble these pieces into the various communication streams. Its primary responsibilities to accomplish this are:

· Tracking the individual communication between applications on the source and destination hosts.

· Segmenting data and managing each piece. The Transport layer protocols describe services that segment this data from the Application layer. This includes the encapsulation required on each piece of data. Each piece of application data requires headers to be added at the Transport layer to indicate to which communication it is associated. 

· Reassembling the segments into streams of application data.

· Identifying the different applications.

Reassembling Segments: At the receiving host, each piece of data may be directed to the appropriate application. Additionally, these individual pieces of data must also be reconstructed into a complete data stream that is useful to the Application layer. The protocols at the Transport layer describe the how the Transport layer header information is used to reassemble the data pieces into streams to be passed to the Application layer.

Identifying the Applications: In order to pass data streams to the proper applications, the Transport layer must identify the target application. To accomplish this, the Transport layer assigns an application an identifier. The TCP/IP protocols call this identifier a port number. Each software process that needs to access the network is assigned a port number unique in that host. This port number is used in the transport layer header to indicate to which application that piece of data is associated.

The Transport layer is the link between the Application layer and the lower layer that are responsible for network transmission. This layer accepts data from different conversations and passes it down to the lower layers as manageable pieces that can be eventually multiplexed over the media.

Applications do not need to know the operational details of the network in use. The applications generate data that is sent from one application to another, without regard to the destination host type, the type of media over which the data must travel, the path taken by the data, the congestion on a link, or the size of the network. 

Additionally, the lower layers are not aware that there are multiple applications sending data on the network. Their responsibility is to deliver data to the appropriate device. The Transport layer then sorts these pieces before delivering them to the appropriate application.

Data Requirements Vary: Because different applications have different requirements, there are multiple Transport layer protocols. For some applications, segments must arrive in a very specific sequence in order to be processed successfully. In some cases, all of the data must be received for any of it to be of use. In other cases, an application can tolerate some loss of data during transmission over the network. 

In today's converged networks, applications with very different transport needs may be communicating on the same network. The different Transport layer protocols have different rules allowing devices to handle these diverse data requirements.

Some protocols provide just the basic functions for efficiently delivering the data pieces between the appropriate applications. These types of protocols are useful for applications whose data is sensitive to delays.

Other Transport layer protocols describe processes that provide additional features, such as ensuring reliable delivery between the applications. While these additional functions provide more robust communication at the Transport layer between applications, they have additional overhead and make larger demands on the network.

Segmentation of the data, in accordance with Transport layer protocols, provides the means to both send and receive data when running multiple applications concurrently on a computer. Without segmentation, only one application, would be able to receive data. 
At the Transport layer, each particular set of pieces flowing between a source application and a destination application is known as a conversation.

To identify each segment of data, the Transport layer adds to the piece a header containing binary data. This header contains fields of bits. It is the values in these fields that enable different Transport layer protocols to perform different functions.

4.1.2. Controlling the Conversations

The primary functions specified by all Transport layer protocols include:

· Segmentation and Reassembly.

· Conversation Multiplexing.

Some protocols at the Transport layer provide:
· Connection-oriented conversations.
· Reliable delivery.
· Ordered data reconstruction.
· Flow control.
[image: image1.png]hitp://curticulum.netacad.net

4 08 Transport Layer

4.1 Roles of the Transport Layer

4.1.2 Controlling the Conversations

CCNA Exploration cisco

Network Fundamentals:

@-

‘The primary functions specified by all Transport layer
protocols include:

Segmentation and Reassembly - Most networks have
limitation on the amount of data that can b included in @
single PDU. The Transport layer divides application data
into blocks of data that are an appropriate size. Atthe
destination, the Transportlayer reassembles the data
before sending itto the destination application or senvice.

Conversation Multplexing - There may be many
applications or senvices running on sach hostin the
netwark. Each ofthese applications or services is
assigned an address known as a port so that the
Transpart layer can determing with which application or
senice the data is identifed

In addition to using the information contained in the
headers, for the basic functions of data segmentation and
reassembly, some protocols atthe Transport layer
provide

+ Connection-oriented conversations

+ Reliable delivery

+ Ordered data reconstruction

+ Flowcantral

Transport Layer Services

INSTANT MULTIPLE WEB PAGES

MESSAGING

EMAIL
1P TELEPHONY

To: you@example.com worr)

From: me@example.com

Subject: E-mail

‘Segmentation allows session STREAMING VIDEO
muliplexing — muliple
appiications can use the netwark

atthe same time. <~

Data scomentaton faciltates data carmiage by the
lower network ayers.

Error checking can be performed on the data in
the segment 1o check f the segment was changed
Guring transmission.

[[rasioingdotafom cucuin etacadnt.

@/ nicio| )3 Explred. ~| o] Mictosoft Po.._ | &3 Windows Liv.. | [ Adobe Rieader [[@ 2 Firetox ~

oot | [N |« & o 1wsnn





· Establishing a Session: The Transport layer can provide this connection orientation by creating a sessions between the applications. 
· Reliable Delivery: The Transport layer can ensure that all pieces reach their destination by having the source device to retransmit any data that is lost.

· Same Order Delivery: By numbering and sequencing the segments, the Transport layer can ensure that these segments are reassembled into the proper order. 
· Flow Control: When Transport layer is aware that network hosts resources are overtaxed, some protocols can request that the sending application reduce the rate of data flow. This is done at the Transport layer by regulating the amount of data the source transmits as a group. Flow control can prevent the loss of segments on the network and avoid the need for retransmission.
[image: image2.png]hitp://curticulum.netacad.net - Mozilla Firefox

4 08 Transport Layer

4.1 Roles of the Transport Layer

4.1.2 Controlling the Conversations

1@

Establishing a Session

‘The Transport ayer can provide this connection
orientation by creating a sessions between the
applications. These connections prepare the applications
to communicate with each other before any data is
transmitied. Within these sessions, the data for a
communication between the two applications can be
closely managed

Refiable Delivery

For many reasons, itis possible for a pisce of data to
become comupted, or lost completely, as tis transrnitted
over the network The Transportlayer can ensure that il
pieces reach their destination by having the source device
to retransrmit any data that s lost.

Same Order Delivery

Because networks may provide multiple routes that can
have diferent transrmission times, data can arfive in the
wrong order. By numbering and sequencing the
segments, the Transportlayer can ensure that these
segments are reassembled into the proper order.

Flow Control

INSTANT
MESSAGING

EMAIL

To: you@example.com
From: me@example.com
‘Subject: Email

Establishing a Session

ensures the application s ready to

receive the data.

‘Same order delivery
ensures data is delvered
sequentialy as it was sent.

[-[Ofx]

CCNA Exploration cisco

Network Fundamentals:

Transport Layer Services

MULTIPLE WEB PAGES

1P TELEPHONY
(voip)

STREAMING VIDEO

Reliable delivery means lost
segments are resent so the data
Is received complee.

Flow Control manages data
delivery f there is congestion on
the host,

[[rasioingdotafom cucuin etacadnt.

2 Inicio| ) 3 Explorador de Wind... <

& Exploraion Index Mo, |[@ http:/Zcuriculum.. Documento! - Microsoft

«@HHO K 152pm




4.1.3. Supporting Reliable Communication

The primary function of the Transport layer is to manage the application data for the conversations between hosts. 
A Transport layer protocol can implement a method to ensure reliable delivery of the data. In networking terms, reliability means ensuring that each piece of data that the source sends arrives at the destination. 
At the Transport layer the three basic operations of reliability are:
· Tracking transmitted data.
· Acknowledging received data.
· Retransmitting any unacknowledged data.
This requires the processes of Transport layer of the source to keep track of all the data pieces of each conversation and the retransmit any of data that did were not acknowledged by the destination. The Transport layer of the receiving host must also track the data as it is received and acknowledge the receipt of the data. 

These reliability processes place additional overhead on the network resources due to the acknowledgement, tracking, and retransmission. To support these reliability operations, more control data is exchanged between the sending and receiving hosts. This control information is contained in the Layer 4 header.

In the context of networking, best-effort delivery is referred to as unreliable, because there is no acknowledgement that the data is received at the destination.

Applications, such as databases, web pages, and e-mail, require that all of the sent data arrive at the destination in its original condition, in order for the data to be useful. Therefore, these applications are designed to use a Transport layer protocol that implements reliability. The additional network overhead is considered to be required for these applications.

Other applications are more tolerant of the loss of small amounts of data. For example, if one or two segments of a video stream fail to arrive, it would only create a momentary disruption in the stream. 
Imposing overhead to ensure reliability for this application could reduce the usefulness of the application. The image in a streaming video would be greatly degraded if the destination device had to account for lost data and delay the stream while waiting for its arrival. It is better to render the best image possible at the time with the segments that arrive and forego reliability. If reliability is required for some reason, these applications can provide error checking and retransmission requests.

[image: image3.png]hitp://curticulum.netacad.net

Mozilla Firefox [-[Ofx]

S “discs

Network Fundarmer

4.1.3 Supporting Reliable Communication .

Transport Layer Protocols

Juiflerent requirements for their data, and therefore
uiflerent Transport protacals have been de

TCPIIP Model

Appiication

Application developers chooss the appropriate Transport Layer protocol based on the nature of
the application.

+ 1P Telephony
+ Streaming Video

« SMTPIPOP (Email)

1 Model
0s! Model s

Application

Required Protocol Propertes Proserai)
. Fast
+ Lowoverhead Session
« Does not require

acknowledgements
+ Does not resend lost
s requires the prac a er ofthe data
burce to keep track of al the data pieces of each « Delvers data as it
ati a athat did arrives
Tran

Required Protocol
Properties
+ Reliable
+ Acknowledge data
+ Resend lost data.
+ Delvers data in
order sent

[[rasioingdotafom cucuin etacadnt.

) tvici] 3 Eplrodor dowind. -

& Exploraion Index Mo, |[@ http:/Zcuriculum.n... ] Documentaf - Microsoft





4.1.4. TCP and UDP

The two most common Transport layer protocols of TCP/IP protocol suite are Transmission Control Protocol (TCP) and User Datagram Protocol (UDP). Both protocols manage the communication of multiple applications. The differences between the two are the specific functions that each protocol implements. 

UDP is a simple, connectionless protocol, described in RFC 768. It has the advantage of providing for low overhead data delivery. The pieces of communication in UDP are called datagrams. These datagrams are sent as "best effort" by this Transport layer protocol. 

Applications that use UDP include:

· Domain Name System (DNS).
· Video Streaming.
· Voice over IP (VoIP).
· Transmission Control Protocol (TCP).
TCP is a connection-oriented protocol, described in RFC 793. TCP incurs additional overhead to gain functions. 
Applications that use TCP are:

· Web Browsers.
· E-mail.
· File Transfers.
[image: image4.png]ol e e
NA CISCO.
4.1.4 TCP and UDP 4 S
I TCP and UDP Headers
agram Protocol (UDP). Both pro
n of multple appiica TCP Segment
pecic functo 81 (0) B (15) Bit (16) BIL(31)
‘Source Port (16) Destination Port (16)
‘Sequence Number (32) T
Acknowledgement Number (32) 20
Header Length (4) Reserved (6) Code Bits 6) Window (16) Bytes
Ghecksum (16) Urgent (16) l
Opions (0 0r 32 f any)
APPLICATION LAYER DATA (Size varies)
UDP Datagram

Bt (0) Bt (15) Bit (16) Bt (31)

Source Port (1) Destination Port (1)
Length (16) Checksum (16)

APPLICATION LAYER DATA (Size varies)

L S (141 by 1 — Q@ ®

[[rasioingdotafom cucuin etacadnt.

@ Esploration Index - Mozi.. | [ @ http://curriculum.n... ] Documento! - Microsaft..| () DR DO <« & %0 K 1055pm.

o] 33 Eplrodor dowind





4.1.5. Port Addressing

The TCP and UDP based services keep track of the various applications that are communicating. To differentiate the segments and datagrams for each application, both TCP and UDP have header fields that can uniquely identify these applications. These unique identifiers are the port numbers.

In the header of each segment or datagram, there is a source and destination port. The source port number is the number for this communication associated with the originating application on the local host. The destination port number is the number for this communication associated with the destination application on the remote host.

When a client application sends a request to a server application, the destination port contained in the header is the port number that is assigned to the service daemon running on the remote host. The client software must know what port number is associated with the server process on the remote host. 
The source port in a segment or datagram header of a client request is randomly generated. As long as it does not conflict with other ports in use on the system, the client can choose any port number. This port number acts like a return address for the requesting application. The Transport layer keeps track of this port and the application that initiated the request so that when a response is returned, it can be forwarded to the correct application. 
The combination of the Transport layer port number and the Network layer IP address assigned to the host uniquely is called a socket. A socket pair, consisting of the source and destination IP addresses and port numbers, is also unique and identifies the conversation between the two hosts.

[image: image5.png]hitp://curticulum.netacad.net

4 08! Transport Layer vl

4.1 Roles of the Transport Layer CCNA Exploration C1SCO

4.15 Port Addressing INetwork Fundamentals

Identitying the Conversations
Port Addressing
Consider the earlier example of a cormputer
simultaneously receiving and sending e-mai,instant
messages, web pages, and a VoIF phone cal

The TOP and UDP based senices keep track ofthe
various applications that are communicating. To
difiersntiate the segments and datagrams for each
application, both TCP and UDP have header fields that

To: you@example.com
From: me@example.com

Subjoct Emai
can uniquely identify these applications. These unique

igentrs ae tne portrumbers.

Inthe header of each segment or datagram, there is a Different

the number for this communication associated with the Protocols '—V
originating applicaion on e focal host The destinaion PortNumbers T oo

o numeristhe number for s communication

associated with the destination application on the remote
Application Applicatin Applcaton
host. Transport Pat Port Port

Fortnumbers are assigned in various ways, depending > 110 80 531
onwhether the message is a request or a response

Wihile server processes have static port numbers

assigned to them, clients dynamically choose a port

number for each conversation.

Data for different applications is directed to the correct appiication because each application has a unique port number.

[[rasioingdotafom cucuin etacadnt.

& Exploraion Index Mo, |[@ http:/Zcuriculum.. Documento! - Microsoft

2 Inicio| ) 3 Explorador de Wind... <

«@ L0 K 15pm





The Internet Assigned Numbers Authority (IANA) assigns port numbers. IANA is a standards body that is responsible for assigning various addressing standards.

There are different types of port numbers:

Well Known Ports (Numbers 0 to 1023) - These numbers are reserved for services and applications. They are commonly used for applications such as HTTP (web server) POP3/SMTP (e-mail server) and Telnet. 
Registered Ports (Numbers 1024 to 49151) - These port numbers are assigned to user processes or applications. These processes are primarily individual applications that a user has chosen to install rather than common applications that would receive a Well Known Port. When not used for a server resource, these ports may also be used dynamically selected by a client as its source port.

Dynamic or Private Ports (Numbers 49152 to 65535) - Also known as Ephemeral Ports, these are usually assigned dynamically to client applications when initiating a connection. 
Some applications may use both TCP and UDP. For example, the low overhead of UDP enables DNS to serve many client requests very quickly. Sometimes, however, sending the requested information may require the reliability of TCP. In this case, the well known port number of 53 is used by both protocols with this service.

[image: image6.png]4 08 Transport Layer
4.1 Roles of the Transport Layer

4.1.5 Port Addressing

CCNA Exploration CISCO
Notwork Fundamentals

“The Intemet Assianed Numbers Authority (ANA) assigns
port numbers. IANA s 3 standards body thatis
responsible for assigning various addressing standards.

There are different types of port numbers:

Well Known Ports (Numbers 0 to 1023) - These numbers
are reserved for senices and applications. They are
commonly used for applications such as HTTP (web
server) POPI/SMTP (e-mail server) and Telnst. By
defining these well-known ports for server applications,
client applications can be programmed to request a
connection to that specifc port and its assosiated service.

Registered Ports (Numbers 1024 to 49151) - These port
numbers are assigned to user processes or applications.

Port Number Range

4915210 65535

Registered TCP Ports: Well Known TCP Ports
MSN Messenger
Alternate HTTP

Alternate HTTP

These processes are primarly indvidual applications that —|
auser has chosen to instal rather than comman
applications that would recsive a Well Known Port When
notused far a server resource, these ports may alsa be
used dynamically selected by a client as its source port

Dynaniic or Private Ports (Nurmibers 4315210 65535) -
Als0 known as Ephemeral Ports, these are usually

assigned dynamicallyto client applications when initiating

a connection.tis notvery cormmon for a lentto connect
o3 senice using a Dynamic or Private Port although =

Click to see the example ports numbers.

Port Numbers

Well Known (Contact) Ports.
Registered Ports
Private and'or Dynamic Ports

P
Telnet

smTP.

HTTP.

POP3

Internet Relay Chat (IRC)
Secure HTTP (HTTPS)

[Lookng s

@ nicio| & gmabia: - Co... | )3 Explorador d,

Resimen Capt | @ Exgoratn e[| @ s/ vl




[image: image7.png]~=lolx|

4.5 Port Addressing PR ok Fundamentals

“The Intemet Assianed Numbers Authority (ANA) assigns
port numbers. IANA s a standards body that is Port Numbers.
responsible for assigning various addressing standards.

Port Number Range Port Group
There are different types of port numbers:

0t0 1023 Well Known (Contact) Ports
Well Known Ports (Nurmbers 0 1 1023) - These nurmbers 1024 1049151
are reserved or senices and applications. They are
cormmonly used for applcations such as HTTP (web
senver) POPIISHTP (e-mail server) and Telnet By

Registered Ports
4915210 65535 Private andior Dynamic Ports

defining these well-known ports for server applications, Registered UDP Ports: Well Known UDP Ports:
client applications can be programmed to request a 1812 RADIUS Authentication Protocol 69 TFIP
connection to that specifc port and its assosiated service. 2000 Cisco SCCP (VoIP) 520 RP

5004 RTP (Voice and Video Transport
Registered Ports (Numbers 1024 to 49151) - These port Protocol)
numbers are assigned to user processes or applications. 5060 SIP (VolP)

These processes are primarly indvidual applications that —|
auser has chosen to instal rather than comman
applications that would recsive a Well Known Port When
notused far a server resource, these ports may alsa be

used dynamically selected by a client as its source port

Dynaniic or Private Ports (Nurmibers 4315210 65535) -

Als0 known as Ephemeral Ports, these are usually

assigned dynamicallyto client applications when initiating

a connection.tis notvery cormmon for a lentto connect
o3 senice using a Dynamic or Private Port although =

Click to see the example ports numbers.

=SSR e = @ ©®

Al contentz c 128Kbps

[Lookng s

e R

(£33 Explora... +| H]Resumenc.. | & Exploration . |[@ filer/7 - Mo...





[image: image8.png]4 08 Transport Layer
4.1 Roles of the Transport Layer

4.1.5 Port Addressing

CCNA Exploration CISCO

Network Fundamentals:

EE

“The Intemet Assianed Numbers Authority (ANA) assigns
port numbers. IANA s 3 standards body thatis
responsible for assigning various addressing standards.

There are different types of port numbers:

Well Known Ports (Numbers 0 to 1023) - These numbers
are reserved for senices and applications. They are
commonly used for applications such as HTTP (web
server) POPI/SMTP (e-mail server) and Telnst. By
defining these well-known ports for server applications,
client applications can be programmed to request a
connection to that specifc port and its assosiated service.

Registered Ports (Numbers 1024 to 49151) - These port
numbers are assigned to user processes or applications.
These processes are primarly indvidual applications that —|
auser has chosen to instal rather than comman
applications that would recsive a Well Known Port When
notused far a server resource, these ports may alsa be
used dynamically selected by a client as its source port

Dynaniic or Private Ports (Nurmibers 4315210 65535) -

Als0 known as Ephemeral Ports, these are usually

assigned dynamicallyto client applications when initiating

a connection.tis notvery cormmon for a lentto connect
o3 senice using a Dynamic or Private Port although =

Port Numbers

Port Number Range Port Group

010 1023 Well Known (Contact) Ports.
102410 49151 Registered Ports
4915210 65535 Private and'or Dynamic Ports

Registered TCPUDP Comimon Ports: Well Known TCPUDP Common Ports:

1433 MsSsoL 53 DNS
2048 WAP (MMS) 161 SNMP
531 AOL Instant Messenger, IRC

Click to see the example ports numbers.

[Lookng s

2 toicto] 5 tinghi Cov. | 5 oboConversa. | (33 Exlorar .+

58] Resumen Captu.. [[@ 2 Firefox -





Netstat is an important network utility that can be used to know which active TCP connections are open and running on a networked host. Netstat lists the protocol in use, the local address and port number, the foreign address and port number, and the state of the connection. 

Unexplained TCP connections can pose a major security threat. This is because they can indicate that something or someone is connected to the local host. Additionally, unnecessary TCP connections can consume valuable system resources thus slowing down the host's performance. 
4.1.6. Segmentation and Reassembly – Divide and Conquer

Some applications transmit large amounts of data - in some cases, many gigabytes. It would be impractical to send all of this data in one large piece. 
Dividing application data into pieces both ensures that data is transmitted within the limits of the media and that data from different applications can be multiplexed on to the media.

In TCP, each segment header contains a sequence number. This sequence number allows the Transport layer functions on the destination host to reassemble segments in the order in which they were transmitted. This ensures that the destination application has the data in the exact form the sender intended.

Although services using UDP also track the conversations between applications, they are not concerned with the order in which the information was transmitted, or in maintaining a connection. There is no sequence number in the UDP header. UDP is a simpler design and generates less overhead than TCP, resulting in a faster transfer of data. 

Information may arrive in a different order than it was transmitted because different packets may take different paths through the network. An application that uses UDP must tolerate the fact that data may not arrive in the order in which it was sent.
[image: image9.png]hitp://curticulum.netacad.net

4 08 Transport Layer
4.1 Roles of the Transport Layer

4.1.6 Segmentation and Reassembly - Divide and Conquer

CCNA Exploration
‘. PR Network Fundamentals:

data is transmitted within the limits of the media and
that data from different applications can be multiplexed
onto the med

TCP and UDP Handle Segmentation Differently.

InTCP, each segment headsr contains a sequence
number. This ssquence number allows the Transport
layer functions on the destination host to reassemble
segments inthe order in which they were ransmitted.
This ensures thatthe destination application has the data
inthe exact form the sender intended.

Although senvices using UDP also track the conversations
between applications, they are not concemed with the
order inwhich the information was transmitted, or in
maintaining a connection. There is no sequence number
inthe UDP header. UDP is a simpler design and
generates less averhead than TCP, resulting in a faster
transfer of data,

Information may arrive in a difierent order than itwas
transmitied because different packets may take different
paths through the network An application that uses UDP
musttolerate the fact that data may not arrive in the order
inwhich itwas sent.

CISCO.

Transport Layer Functions
APPLICATION LAYER DATA
The Transport layer
divides the data ino. Plecs 1 Plece 2 Plece 3
pieces and adds a header
for delvery over the. UDP Datagram or TCP Segment
network.
Piece 1 Plece 1
-E feet
-E feee?
TCP Header provides for
+ Source & destinaton (ports)
UDP Hesder provides for: + Sequencing for same order delivery

+ Source and destination

(ports)

« Acknowledgement of
segments

received

+ Flow control and congestion

management

[[rasioingdotafom cucuin etacadnt.

2 Inicio| ) 3 Explorador de Wind... <

& Exploraion Index Mo, |[@ http:/Zcuriculum.n... ] Documentaf - Microsoft





4.2.1. TCP – Making Conversations Reliable

The key distinction between TCP and UDP is reliability. The reliability of TCP communication is performed using connection-oriented sessions. Before a host using TCP sends data to another host, the Transport layer initiates a process to create a connection with the destination. This connection enables the tracking of a session, or communication stream between the hosts. This process ensures that each host is aware of and prepared for the communication. A complete TCP conversation requires the establishment of a session between the hosts in both directions.

After a session has been established, the destination sends acknowledgements to the source for the segments that it receives. These acknowledgements form the basis of reliability within the TCP session. As the source receives an acknowledgement, it knows that the data has been successfully delivered and can quit tracking that data. If the source does not receive an acknowledgement within a predetermined amount of time, it retransmits that data to the destination. 

Part of the additional overhead of using TCP is the network traffic generated by acknowledgements and retransmissions. The establishment of the sessions creates overhead in the form of additional segments being exchanged. There is also additional overhead on the individual hosts created by the necessity to keep track of which segments are awaiting acknowledgement and by the retransmission process.

This reliability is achieved by having fields in the TCP segment, each with a specific function, as shown in the figure. These fields will be discussed later in this section.

[image: image10.png]http://curriculum.netacad.net - Mozilla Firefox =[]

Bt el [
CISCO.
4.2.1 TCP - Making Conversations Reliable y _
TCP Segment Header Fields
8ito 15 a1

Source Port Number Destination Port Number

I Sequence Number

thatthe data ha

Acknowledgement Number

H.Length (Reserved) Flags Window Size

TCP Checksum Urgent Pointer

Options (if any)

‘The fields of the TCP header enable TCP to provide connection-oriented, reliable data
communications.

Roll over each fleld to see its function.

=R R 2 e ey — - @ ©®

Translering deta from curiculm netacad et

)it 13 Exort do| @ Exprsinindon- [ et rearios | ) Dooirertot Wi | 5 wiows et | IO« & %0 % oowm





4.2.2. TCP Server Processes

Each application process running on the server is configured to use a port number. An individual server cannot have two services assigned to the same port number within the same Transport layer services. When an active server application is assigned to a specific port, that port is considered to be "open" on the server. This means that the Transport layer accepts and processes segments addressed to that port. Any incoming client request addressed to the correct socket is accepted and the data is passed to the server application. There can be many simultaneous ports open on a server, one for each active server application. It is common for a server to provide more than one service, such as a web server and an FTP server, at the same time.

[image: image11.png]2) hitp://curticulum. netacad.net - Mozilla Firefox

4.2.2 TCP Server Processes

Each application process running n the server is
configured to use a port number, sither by default or
manually by  syster administrator. An individual server
cannot have two services assigned to the same port
number within the same Transport layer services. 4
host running a web server application and a fle transfer
application cannot have both configured to use the same
port for example, TCP port 8080). When an active server
application is assigned to a specific port that portis
considersdto be "open’ on the server. This means that
the Transportlayer accepts and processes segments
addressed to that port Any incoming client request
addressed 1o the correct socket s accepted and the data
is passedto the server application. There can be many
simultansous ports open on a server, one for each active
server application. Itis comrmon for 3 server to provide
morethan one service, such as a web server and an FTP.
server, atthe same time.

One wayto improve security on a server is to restrict
server access to onlythose ports assosiated with the
senices and applications that should be accessible to
authorized requestors.

‘The figure shows the typical allocation of source and
destination ports in TCP clientiserver operations.

[-[Ofx]

il
CISCO.

Clients Sending TCP Requests

HTTP response: Server SMTP Response:
Source Port 80 Source Port 25
Destination Port 49152 Destinaion Port 51152
Client 1 HTTP: Port 80 Client 2
SMTP: Port 25

Client requests to TCP
server

SMTP Request;
Server response to TCP clients use | Source Port: 51152

random port numbers as the Destination Port: 25
destination port.

Click to see the the types of ports used.

S

HTTP Request;

Source Port: 49152

Destination Port: 80

Request Source Ports

[[rasioingdotafom cucuin etacadnt.

2 Inicio| ) 3 Exploradorde...+| @ Exploraton Incet.. [ @ http:/7curricu... ] Documentot -Mi.. | 3 Windows Live M. | R RELEE

SO |« & %9 K 1103em





4.2.3. TCP Connection Establishment and Termination

When two hosts communicate using TCP, a connection is established before data can be exchanged. After the communication is completed, the sessions are closed and the connection is terminated. The connection and session mechanisms enable TCP's reliability function.

The host tracks each data segment within a session and exchanges information about what data is received by each host using the information in the TCP header. 

To establish the connection, the hosts perform a three-way handshake. Control bits in the TCP header indicate the progress and status of the connection. The three-way handshake:
· Establishes that the destination device is present on the network.
· Verifies that the destination device has an active service and is accepting requests on the destination port number that the initiating client intends to use for the session.
· Informs the destination device that the source client intends to establish a communication session on that port number.
In TCP connections, the host serving as a client initiates the session to the server. The three steps in TCP connection establishment are:

1. The initiating client sends a segment containing an initial sequence value, which serves as a request to the server to begin a communications session.

2. The server responds with a segment containing an acknowledgement value equal to the received sequence value plus 1, plus its own synchronizing sequence value. The value is one greater than the sequence number because the ACK is always the next expected Byte or Octet. This acknowledgement value enables the client to tie the response back to the original segment that it sent to the server. 

3. Initiating client responds with an acknowledgement value equal to the sequence value it received plus one. This completes the process of establishing the connection.

To understand the three-way handshake process, it is important to look at the various values that the two hosts exchange. Within the TCP segment header, there are six 1-bit fields that contain control information used to manage the TCP processes. Those fields are: 

· URG - Urgent pointer field significant

· ACK - Acknowledgement field significant

· PSH - Push function

· RST - Reset the connection

· SYN - Synchronize sequence numbers

· FIN - No more data from sender

These fields are referred to as flags, because the value of one of these fields is only 1 bit and, therefore, has only two values: 1 or 0. When a bit value is set to 1, it indicates what control information is contained in the segment.

4.2.5. TCP Session Termination

To close a connection, the FIN (Finish) control flag in the segment header must be set. To end each one-way TCP session, a two-way handshake is used, consisting of a FIN segment and an ACK segment. Therefore, to terminate a single conversation supported by TCP, four exchanges are needed to end both sessions. Note: In this explanation, the terms client and server are used in this description as a reference for simplicity, but the termination process can be initiated by any two hosts that complete the session:

1. When the client has no more data to send in the stream, it sends a segment with the FIN flag set.

2. The server sends an ACK to acknowledge the receipt of the FIN to terminate the session from client to server.

3. The server sends a FIN to the client, to terminate the server to client session.

4. The client responds with an ACK to acknowledge the FIN from the server.

When the client end of the session has no more data to transfer, it sets the FIN flag in the header of a segment. Next, the server end of the connection will send a normal segment containing data with the ACK flag set using the acknowledgment number, confirming that all the bytes of data have been received. When all segments have been acknowledged, the session is closed.

The session in the other direction is closed using the same process. The receiver indicates that there is no more data to send by setting the FIN flag in the header of a segment sent to the source. A return acknowledgement confirms that all bytes of data have been received and that session is, in turn, closed.

It is also possible to terminate the connection by a three-way handshake. When the client has no more data to send, it sends a FIN to the server. If the server also has no more data to send, it can reply with both the FIN and ACK flags set, combining two steps into one. The client replies with an ACK.

[image: image12.png]4 08! Transport Layer vl

4.2 The TCP Protocol - Communicating with Reliability CCNA Exploration (E)

423 TCP Connection Establishment and Termination L Trorental

When two hosts communicate using TCP, 3 connection is

established before data can be exchanged. After the TCP Connection Establishment and Termination

communication is completed, the sessions are closed A B
TCP connection. (SEQ=100 CTL=SYN) SYN received

The hostiracks each data segmentwithin a session and Send SYN.ACK .
exchanges information about what data is received by SYN received (SEQ=300 ACK=101 CTL=SYN.ACK)

each host using the information in the TCP header. Established
(SEQ=101 ACK=301 CTL=ACK

Each connection represents bwo one-way communication
strearns, or sessions. To establish the connection, the
hosts perform a three-way handshake. Controlbits in the
TCP header indicate the progress and status ofthe
connection. The three-way handshake Asends ACK response to B.
- Establishes thatthe destination device is

present on the network - SYN ACK - - n

intends to use for the session

+ Informs the destination device thatthe source Click to see the steps.
clientintends to establish a communication
sesinn anthat nart mirmner

ctl = Which control bits in the TCP header are set to 1

o =]

1 1nicio ) Aves checar a... | () Activdades, T... | [} Mis documentos | [#]3 Mirosot ... «|[@ 2 Firefon - 3] Mcrosoft Exc... | [« (o @@ &K osi10pm.




 [image: image13.png]4 08 Transport Layer

4.2.3 TCP Connection Establishment and Termination

4.2 The TCP Protocol - Communicating with Reliability

CCNA Exploration
Network Fundamentals:

When two hosts communicate using TCP, 3 connection is
established before data can be exchanged. After e
communication is completed, the sessions are closed
and the connection is terminated. The connection and
session mechanisms enahle TOP's reliabillty function,

See the figure for the steps to establish and terminate a
TCP connection.

‘The host tracks each data segment within a session and
exchanges information about what data is received by
each host using the information in the TCP header.

Each connection represents two one-way communication
streams, or sessions. Ta establish the connection, the
hosts perform a thres-way handshake. Control bits in the
TCP header indicate the progress and status ofthe
connection. The three-way handshake:
+ Establishes that the destination device is
present on the network
+ Verifies thatthe destination device has an active
senice and is accepting requests on the
destination port number that the initiaing client
intends to use for the session
+ Informs the destination device that the source
client intends to establish a communication
spasinn nn that nor nirmher

| - -

TCP Connection Establishment and Termination
A B

. Send FIN

send Ak (2)
ACK received

FIN received

FIN received

. Send ACK
ACK received

Asends ACK response to B.

L T 1 1

kto see the steps.

vl
CIsco

[Lookng s

=3}

@ inicio|| 34 windows L.+ () res checar a... | () ctividades, T...| () Mis documentos | [#]3 Mcrosoft .. +[[@2 Firefox - (3] Mirosoft Exc... | [« (@@ B K 09:09pm,





4.3.1. TCP Segment Reassembly

Sequence numbers are assigned in the header of each packet to achieve this goal. 

During session setup, an initial sequence number (ISN) is set. This initial sequence number represents the starting value for the bytes for this session that will be transmitted to the receiving application. As data is transmitted during the session, the sequence number is incremented by the number of bytes that have been transmitted. This tracking of data byte enables each segment to be uniquely identified and acknowledged. Missing segments can be identified. 

Segment sequence numbers enable reliability by indicating how to reassemble and reorder received segments. 

The receiving TCP process places the data from a segment into a receiving buffer. Segments are placed in the proper sequence number order and passed to the Application layer when reassembled. Any segments that arrive with noncontiguous sequence numbers are held for later processing. Then, when the segments with the missing bytes arrive, these segments are processed.

[image: image14.png]B “discs

43.1 TCP Segment Reassembly et

sequencing Segments to Order Transmitted|

nd tata Using T TCP Segments Are Re-Ordered at the Destination

attheir destination aut of order. For the original

Different segments may take
different routes.

p, aninitial

auence number repr "
Segment 1 Segment  TCPre-  Segmentq
—
Data orders the

Segment2  Havngisken  Segment2  Co T K Segment2

different rout "
Datais  Segment3 o the original  Segment 3

divided into dostat order.
segments., | Segment4 lestination, Segment 5 Segment 4

segments

Segment5 | amveoutof | Segmentd Segment5

order.
 Segment6 Segment 3

[[rasioingdotafom cucuin etacadnt.

2/ nicio| ) 3 Explorador de. | (& Evploaton inie..|[@ http:/7curricu.. ] Documenta -Mi. | &3 windows Live .. | [





4.3.2. TCP Acknowledgement with Windowing

One of TCP's functions is making sure that each segment reaches its destination. The TCP services on the destination host acknowledge the data that it has received to the source application. 

The segment header sequence number and acknowledgement number are used together to confirm receipt of the bytes of data contained in the segments. The sequence number is the relative number of bytes that have been transmitted in this session plus 1 (which is the number of the first data byte in the current segment). TCP uses the acknowledgement number in segments sent back to the source to indicate the next byte in this session that the receiver expects to receive. This is called expectational acknowledgement. 

The source is informed that the destination has received all bytes in this data stream up to, but not including, the byte indicated by the acknowledgement number. The sending host is expected to send a segment that uses a sequence number that is equal to the acknowledgement number.

Each connection is actually two one-way sessions. Sequence numbers and acknowledgement numbers are being exchanged in both directions.

In the example in the figure, the host on the left is sending data to the host on the right. It sends a segment containing 10 bytes of data for this session and a sequence number equal to 1 in the header. 

The receiving host on the right receives the segment at Layer 4 and determines that the sequence number is 1 and that it has 10 bytes of data. The host then sends a segment back to the host on the left to acknowledge the receipt of this data. In this segment, the host sets the acknowledgement number to 11 to indicate that the next byte of data it expects to receive in this session is byte number 11.

When the sending host on the left receives this acknowledgement, it can now send the next segment containing data for this session starting with byte number 11.

The amount of data that a source can transmit before an acknowledgement must be received is called the window size. Window Size is a field in the TCP header that enables the management of lost data and flow control.

[image: image15.png]4.3.2 TCP Acknowledgement with Windowing B

Acknowledgement of TCP Segments

Source Port | Destination | Sequence | Acknowledgement

Port Number Numbers
Ireceived 10 bytes
Start with byte #1, starting with byte #1
1am sending 10 bytes. 1 expect byte #11 next.
Network
5 <

Source _Dest. _Seq. _Ack
08 | 2 | 1

10 bytes.
Source _Dest__Seq__Ack.
-« = m 1

Source _Dest _Seq. Ack.

wn = v -] ——
e g 1

[[rasioingdotafom cucuin etacadnt.

12 Inicio| ) 3 Explorador de...+| @ Evploraton Inces.. [ @ http:/7curricu... ] Documentol -Mi. | &3 Windows Live M, «@ WO K 115m





4.3.3. Handling Segement Loss

A destination host service using TCP usually only acknowledges data for contiguous sequence bytes. If one or more segments are missing, only the data in the segments that complete the stream are acknowledged. 

When TCP at the source host has not received an acknowledgement after a predetermined amount of time, it will go back to the last acknowledgement number that it received and retransmit data from that point forward.

For a typical TCP implementation, a host may transmit a segment, put a copy of the segment in a retransmission queue, and start a timer. When the data acknowledgment is received, the segment is deleted from the queue. If the acknowledgment is not received before the timer expires, the segment is retransmitted. 

Hosts today may also employ an optional feature called Selective Acknowledgements. If both hosts support Selective Acknowledgements, it is possible for the destination to acknowledge bytes in discontinuous segments and the host would only need to retransmit the missing data.

4.3.4. TCP Congestion Control – Minimizing Segment Loss

Flow control assists the reliability of TCP transmission by adjusting the effective rate of data flow between the two services in the session. When the source is informed that the specified amount of data in the segments is received, it can continue sending more data for this session.

This Window Size field in the TCP header specifies the amount of data that can be transmitted before an acknowledgement must be received. The initial window size is determined during the session startup via the three-way handshake.

TCP feedback mechanism adjusts the effective rate of data transmission to the maximum flow that the network and destination device can support without loss. 
During the delay in receiving the acknowledgement, the sender will not be sending any additional segments for this session. In periods when the network is congested or the resources of the receiving host are strained, the delay may increase. As this delay grows longer, the effective transmission rate of the data for this session decreases. The slowdown in data rate helps reduce the resource contention.
[image: image16.png]434 TCP Congestion Control - Minimizing Segment Loss @ 2 |usssctusions

TCP Segment Acknowledgement and Window Size

] 5
Sender Window size = 3000 Receiver

1500 bytes.
Sequence number 1 \—————"—3»| Recelve 1- 1500
1500 bytes.
‘Sequence number 1501 ———20 B 3 pcgive 1501 - 3000

Receive Acknowledge | <€ Acknowledgement number 3001

1500 bytes.

Sequence number 3001 ' ————————"—— 3 Recelve 3001 - 4500
1500 bytes.

Sequence number 4501 | ———————~—"——3 Recelve 4501 - 6000

Receive Acknowledge | <€ Acknowledgement number 6001

nder will not be

The window size determines the number of bytes sent before an
acknowledgment is expected.
The acknowledgement number is the number of the next expected
byte.

[[rasioingdotafom cucuin etacadnt.

12 Inicio| ) 3 Explorador de...+| @ Evploraton Inces.. [ @ http:/7curricu... ] Documentol -Mi. | &3 Windows Live M, «@ WO K 1M17pm





Another way to control the data flow is to use dynamic window sizes. When network resources are constrained, TCP can reduce the window size to require that received segments be acknowledged more frequently. 
The TCP receiving host sends the window size value to the sending TCP to indicate the number of bytes that it is prepared to receive as a part of this session. 
After periods of transmission with no data losses or constrained resources, the receiver will begin to increase the window field. This reduces the overhead on the network because fewer acknowledgments need to be sent. Window size will continue to increase until there is data loss, which will cause the window size to be decreased.

This dynamic increasing and decreasing of window size is a continuous process in TCP, which determines the optimum window size for each TCP session. 
4.4.1. UDP – Low Overhead vs. Reliability

UDP is a simple protocol that provides the basic Transport layer functions. It much lower overhead than TCP, since it is not connection-oriented and does not provide the sophisticated retransmission, sequencing, and flow control mechanisms.

Although the total amount of UDP traffic found on a typical network is often relatively low, key Application layer protocols that use UDP include:
· Domain Name System (DNS)

· Simple Network Management Protocol (SNMP)

· Dynamic Host Configuration Protocol (DHCP)

· Routing Information Protocol (RIP)

· Trivial File Transfer Protocol (TFTP)

· Online games

[image: image17.png]il
CISCO.

4.4.1 UDP - Low Overhead vs. Reliability

UDP Low Overhead Data Transport
! Network !

23 2
Sender Receiver
r

ik D) R

UDP does not establish a connection
before sending data.

UDP provides low overhead data transport because it has a small datagram
header and no network management traffic.

SR BB (a1 by s m— — L — @ ©

[[rasioingdotafom cucuin etacadnt.

) tnicio| |53 Explorador de..-| & Explraton nex..|[@ it/ 7curricu_ ] Documento1 -i. | 3 windows Live .. | [ RS DK < & %D K 11:195m





4.4.2. UDP Datagram Reassembly

Because UDP is connectionless, sessions are not established before communication takes place as they are with TCP. UDP is said to be transaction-based. In other words, when an application has data to send, it simply sends the data.

Many applications that use UDP send small amounts of data that can fit in one segment. However, some applications will send larger amounts of data that must be split into multiple segments. The UDP PDU is referred to as a datagram.

UDP has no way to reorder the datagrams into their transmission order. UDP simply reassembles the data in the order that it was received and forwards it to the application. If the sequence of the data is important to the application, the application will have to identify the proper sequence of the data and determine how the data should be processed.

[image: image18.png]4 08! Transport Layer vl
4.4 The UDP Protocol - Communicating with Low Overhead CCNA Exploration Cisco.
Network Fundamentals

4.4.2 UDP Datagram Reassembly

Because UDP is connectionless, sessions are not
established before communication takes place as they UDP: Connectionless and Unreliable
are with TCP. UDP is said to be transaction-hased. In Diffrent datagrams may take
other words, when an application has datato send, it

different routes.
simply sends the dafa N

Many applications that use UDP send small amounts of
data that can fitin one segment. However, some
applications will send larger amounts of data that must
be splitinto multiple segments The UDP PDU is referred
to as a datagram, although the terms segmentand
catagram are somefimes used interchangeatly to

deserine a Transport ayer PDU.
© Datagam1

| Datagam1  Outof order

When muliple datagrams are sentto a destination, they et > datagrams are

eyt et paihs and o n o rcr [osagem2 | | vavingtakon | [Daagm2 | T

UDP does not keep track of sequence numbers the way Data is Datagram 3 different routes

TCP does. UDP has noway t reorder the datagrams info P, 1o tothe Loet gt

theirransmission order. See the figure dtogjams, | Datagram 4 destination, | Datagrams|| Lostdatagrams
datagrams are not re-sent.

Therefore, UDP simply reassembles the data inthe order | Datagam5’ | amive outof Datagram 4.

thatitwas received and forwards itto he application. fthe ord

seauence ofth data 5 mporantts e applcaton,the | oatagrame |

application will have to identity the proper sequence ofthe
data and determine how the data should be processed.

[[rasioingdotafom cucuin etacadnt.

& Exploraion Index Mo, |[@ http:/Zcuriculum.. Documento! - Microsoft

2 Inicio| ) 3 Explorador de Wind.





4.4.3. UDP Server Processes and Requests

Like TCP-based applications, UDP-based server applications are assigned Well Known or Registered port numbers. When these applications or processes are running, they will accept the data matched with the assigned port number. When UDP receives a datagram destined for one of these ports, it forwards the application data to the appropriate application based on its port number.

4.4.4. UDP Client Processes

As with TCP, client/server communication is initiated by a client application that is requesting data from a server process. The UDP client process randomly selects a port number from the dynamic range of port numbers and uses this as the source port for the conversation. The destination port will usually be the Well Known or Registered port number assigned to the server process. 

Randomized source port numbers also help with security. If there is a predictable pattern for destination port selection, an intruder can more easily simulate access to a client by attempting to connect to the port number most likely to be open.

Because there is no session to be created with UDP, as soon as the data is ready to be sent and the ports identified, UDP can form the datagram and pass it to the Network layer to be addressed and sent on the network.

Remember, once a client has chosen the source and destination ports, the same pair of ports is used in the header of all datagrams used in the transaction. For the data returning to the client from the server, the source and destination port numbers in the datagram header are reversed.

[image: image19.png]il
NA CISCO.
4.4.4 UDP Client Processes @ 2 Ml
mrmunica
ting data lients Sending UDP Requests
Server DNS response Senver Server RADIUS Response:
Desunalmn Port 49152 Destination Port 51152
cw t1 Client 2
[Randomized saurce port numbers also help with secur = DNS: Port 53 e
rtnere is a predictable pattern or destination p RADIUS: Port 1812 '
lection, an infruder can more &

Server response to UDP clients use random port
numbers as the destination port.

Client 1 waiting for server Client 2 waiting for server
DN response on Po RADIUS response
49152 on Port 51152

ination port numbers in the datagram heaer ar
Click to see the the types of ports used.

SR SR [@aa e — L — @ ©

[[rasioingdotafom cucuin etacadnt.

@ Esploration Index - Mozi.. | [ @ http://curriculum.n... ] Documento! - Microsaft..| () DR DO <« & %0 K 11:23pm

o] 33 Eplrodor dowind





